| Haftka[4] | Variable complexity modelling, multiplicative scaling | Not adaptive scaling factor | Correction/scaling/bridging function |
| --- | --- | --- | --- |
| Alexandrov[5] | Approximation Management Framework (AMF), 1st order additive and multiplicative scaling | Over reliance on low fidelity model, misguided search |  |
| Gano[9] | Kriging based scaling function, hybrid scaling method | No improvement in quality of results, significant low fidelity evaluation, expensive kriging model |  |
| Eldred[8] | 2nd order additive, multiplicative and combined correction, | Effective upon conditions, not adaptive, less dimensional problem |  |
| Qian[12] | Gaussian process modelling, Bayesian adjustment, does not suffer from curse of dimensionality | Prior knowledge of mean of approximate, applied to only two fidelity levels |  |
| March[16] | Co-kriging Bayesian model calibration, trust region framework, gradient based adjustment | No adaptive selection of fidelity, only recommended in less dimensional problem |  |
| Sun[18] | Comparison of different surrogate models: RSM, Kriging, RBF, SVR, ANN, more sample points incorporation without significant increase in computational cost, | Multiobjective posed as single objective, non-linear problem handling |  |
| Kim[22] | High fidelity prediction based on low fidelity knowledge, KBNN | Many different input pattern can produce same outputs, less resolution in regions of interest, less accuracy, extrapolation beyond training data is not possible | Knowledge based |
| Wang[21] | High fidelity prediction based on low fidelity knowledge, KBNN | Many different input pattern can produce same outputs, less resolution in regions of interest, less accuracy, extrapolation beyond training data is not possible |  |
| Madsen[58] | Provably convergent space mapping | Poor convergence, dependent on many parameters, overhead calculation at each iteration, constraint problem is not solved | Space mapping |
| Robinson[59] | Trust region and space mapping, | 1st order accuracy is mandatory, dependent on many parameters, overhead calculation at each iteration, no constraint handling |  |
| Forrester[35] | Co-kriging with Bayesian model update | Higher dimension, limited sample size, | Approximation/surrogate (unified model) Gradient based |
| Lim[38] | Fidelity choice based on correlation, local search from most appropriate point of the most suitable fidelity level, introduced rank correlation coefficient to judge the quality of approximation | Too many parameters, local search might misguide |  |
| Balabanov[31] | No need for correlation, gradient based search, any number of variables, | No adaptive way to select low and high fidelity for evaluation, low and high fidelity should have same variables, chance of being misguided, no automated gradient calculation, limited to linear models |  |
| Kampolis[32] | Applicable to single/multi-objective constrained/unconstrained problem, gradient based search | Limited to two fidelity levels, too many parameters, building local metamodel at each iteration might be expensive |  |
| Koziel and Leifsson[2013] |  |  |  |
| Koziel[48] |  |  | Switching fidelity |
| Jones[51] |  |  |  |